Dynamic IPv6 Routing using RIPng Protocol in Cisco Packet Tracer

Project Report

Name: Symbol Rani & Rimsha katpar

Course: Computer Netwroks & Data Communication

Submitted to: Sir Muhammad Bux

Date: May 29, 2026

## Table of Contents

| # | Topic | Page |
| --- | --- | --- |
| 1 | Introduction | 2 |
| 2 | Objectives | 2 |
| 3 | Technologies Used | 3 |
| 4 | Purpose of Technologies Used | 3 |
| 5 | Network Topology | 5 |
| 6 | IPv6 Addressing Table | 6 |
| 7 | Hardware & Module Configuration | 7 |
| 8 | Router Configuration | 8 |
| 9 | RIPng Configuration | 10 |
| 10 | PC Configuration | 11 |
| 11 | Testing and Verification | 12 |
| 12 | Troubleshooting | 13 |
| 13 | Conclusion | 14 |
| 14 | References | 14 |

## 1. Introduction

This project presents the design and implementation of an IPv6-based routed network using Cisco Packet Tracer. The network simulates a multi-router environment where multiple end devices communicate across different IPv6 subnets using dynamic routing.

The project includes important networking technologies such as IPv6 addressing, RIPng (Routing Information Protocol next generation) , static IPv6 configuration, and serial WAN connections. Three routers were interconnected using serial DCE links to simulate a wide area network (WAN) environment.

RIPng was configured as the dynamic routing protocol to automatically share IPv6 route information between routers, eliminating the need for manual static routes. End devices (PCs) were configured with IPv6 addresses and default gateways to enable cross-network communication.

The project demonstrates how IPv6 and RIPng can be implemented to create a scalable, efficient, and modern network infrastructure suitable for enterprise environments transitioning from IPv4 to IPv6.

## 2. Objectives

The objectives of this project are:

| 1 | To design and simulate an IPv6-based routed network using Cisco Packet Tracer |
| --- | --- |
| 2 | To configure IPv6 addresses on routers and end devices |
| 3 | To implement serial WAN connections between routers using Serial DCE cables |
| 4 | To configure RIPng for dynamic IPv6 routing |
| 5 | To enable inter-network communication across different IPv6 subnets |
| 6 | To verify routing tables and route propagation using show ipv6 route |
| 7 | To test connectivity using ICMPv6 ping between devices in different networks |
| 8 | To understand the differences between IPv4 and IPv6 routing protocols |

## 3. Technologies Used

| Technology | Purpose |
| --- | --- |
| IPv6 Addressing | Next-generation IP addressing for end devices and routers |
| RIPng | Dynamic routing protocol for IPv6 networks |
| Serial DCE | WAN connection between routers with clock rate configuration |
| GigabitEthernet | LAN connections between routers and end devices |
| ICMPv6 | IPv6 ping testing and verification |
| Cisco Packet Tracer | Network simulation and testing environment |
| SLAAC / Static IPv6 | IPv6 address assignment for end devices |

## 4. Purpose of Technologies Used

### IPv6 (Internet Protocol version 6)

IPv6 was used as the primary addressing scheme for all routers and end devices in the network. IPv6 provides a significantly larger address space compared to IPv4 and eliminates the need for NAT (Network Address Translation).

Benefits:

- Larger address space (128-bit addresses)

- Built-in IPsec support

- Simplified header format

- Autoconfiguration capabilities (SLAAC)

- No need for NAT

### RIPng (Routing Information Protocol next generation)

RIPng is the IPv6 version of RIP (Routing Information Protocol). It was used as the dynamic routing protocol to automatically exchange IPv6 route information between routers.

Benefits:

- Automatically learns routes to remote networks

- Eliminates manual static route configuration

- Uses UDP port 521 for communication

- Simple configuration and easy to troubleshoot

- Suitable for small to medium IPv6 networks

### Serial DCE (Data Communications Equipment)

Serial DCE cables were used to connect routers together, simulating WAN links between different geographical locations. The DCE end requires a clock rate to be set for synchronization.

Benefits:

- Simulates real-world WAN connections

- Requires clock rate configuration (practical skill)

- Supports point-to-point connectivity

- Cost-effective in simulation environments

### GigabitEthernet

GigabitEthernet interfaces were used to connect routers to end devices (PCs), providing high-speed LAN connectivity.

Benefits:

- High bandwidth (1 Gbps)

- Standard for enterprise LAN connections

- Reliable and widely supported

### ICMPv6 (Internet Control Message Protocol version 6)

ICMPv6 was used for testing connectivity between devices using the ping command. It also plays a critical role in neighbor discovery and autoconfiguration.

Benefits:

- Essential for network troubleshooting

- Verifies end-to-end connectivity

- Confirms routing protocol functionality

### Cisco Packet Tracer

Cisco Packet Tracer was used as the network simulation tool for designing, configuring, and testing the RIPng IPv6 network.

Benefits:

- Safe testing environment without physical hardware

- Realistic Cisco IOS command-line interface

- Easy visualization of network topology

- Excellent learning tool for certification preparation

## 5. Network Topology

The network topology consists of:

| Component | Quantity |
| --- | --- |
| Routers (ISR 4321 / 2911) | 3 |
| PCs | 3 |
| Serial DCE Links | 2 |
| GigabitEthernet Links | 3 |

### Network Topology

### IPv6 Subnets Used

| Subnet | Purpose | Network Address |
| --- | --- | --- |
| Subnet A | PC0 Network | 2001:db8:1::/64 |
| Subnet B | PC1 Network | 2001:db8:2::/64 |
| Subnet C | PC2 Network | 2001:db8:3::/64 |
| Subnet D | R0 ↔ R1 Link | 2001:db8:12::/64 |
| Subnet E | R1 ↔ R2 Link | 2001:db8:23::/64 |

## 6. IPv6 Addressing Table

### Router Interfaces

| Router | Interface | IPv6 Address | Connected To |
| --- | --- | --- | --- |
| Router0 | GigabitEthernet0/0/0 | 2001:db8:1::1/64 | PC0 |
| Router0 | Serial0/1/0 | 2001:db8:12::1/64 | Router1 |
| Router1 | GigabitEthernet0/0/0 | 2001:db8:2::1/64 | PC1 |
| Router1 | Serial0/1/0 | 2001:db8:12::2/64 | Router0 |
| Router1 | Serial0/1/1 | 2001:db8:23::1/64 | Router2 |
| Router2 | GigabitEthernet0/0/0 | 2001:db8:3::1/64 | PC2 |
| Router2 | Serial0/1/0 | 2001:db8:23::2/64 | Router1 |

### End Devices (PCs)

| Device | Interface | IPv6 Address | Link Local Address | Default Gateway (Link Local) |
| --- | --- | --- | --- | --- |
| PC0 | FastEthernet0 | 2001:DB8:1:0:2E0:B0FF:FE86:5EA7 | FE80::2E0:B0FF:FE86:5EA7 | FE80::204:9AFF:FEA2:EE01 |
| PC1 | FastEthernet0 | 2001:DB8:2:0:20A:41FF:FEE2:D773 | FE80::20A:41FF:FEE2:D773 | FE80::204:9AFF:FEA2:EE01 |
| PC2 | FastEthernet0 | 2001:DB8:3:0:2E0:A3FF:FE06:6410 | FE80::2E0:A3FF:FE06:6410 | FE80::204:9AFF:FEA2:EE01 |

## 7. Hardware & Module Configuration

### NIM-2T Module Installation

The ISR 4321 routers do not have built-in serial ports. The NIM-2T (2-port Serial Network Interface Module) was added to each router to enable serial WAN connectivity.

Steps to add NIM-2T:

- Click on Router → Physical tab

- Power OFF the router using the power switch

- Drag NIM-2T module from the modules list into an available slot

- Power ON the router

- Verify serial ports appear using show ip interface brief

## 8. Router Configuration

### Router0 Configuration

cisco

enable

configure terminal

hostname R0

! Enable IPv6 routing

ipv6 unicast-routing

! Configure GigabitEthernet interface to PC0

interface gigabitethernet0/0/0

ipv6 address 2001:db8:1::1/64

ipv6 rip RIPNG enable

no shutdown

exit

! Configure Serial interface to Router1

interface serial0/1/0

ipv6 address 2001:db8:12::1/64

ipv6 rip RIPNG enable

clock rate 64000

no shutdown

exit

! Enable RIPng

ipv6 router rip RIPNG

exit

! Save configuration

write memory

### Router1 Configuration

cisco

enable

configure terminal

hostname R1

! Enable IPv6 routing

ipv6 unicast-routing

! Configure GigabitEthernet interface to PC1

interface gigabitethernet0/0/0

ipv6 address 2001:db8:2::1/64

ipv6 rip RIPNG enable

no shutdown

exit

! Configure Serial interface to Router0

interface serial0/1/0

ipv6 address 2001:db8:12::2/64

ipv6 rip RIPNG enable

no shutdown

exit

! Configure Serial interface to Router2

interface serial0/1/1

ipv6 address 2001:db8:23::1/64

ipv6 rip RIPNG enable

clock rate 64000

no shutdown

exit

! Enable RIPng

ipv6 router rip RIPNG

exit

! Save configuration

write memory

### Router2 Configuration

cisco

enable

configure terminal

hostname R2

! Enable IPv6 routing

ipv6 unicast-routing

! Configure GigabitEthernet interface to PC2

interface gigabitethernet0/0/0

ipv6 address 2001:db8:3::1/64

ipv6 rip RIPNG enable

no shutdown

exit

! Configure Serial interface to Router1

interface serial0/1/0

ipv6 address 2001:db8:23::2/64

ipv6 rip RIPNG enable

no shutdown

exit

! Enable RIPng

ipv6 router rip RIPNG

exit

! Save configuration

write memory

## 9. RIPng Configuration

RIPng was enabled on all routers using the following commands:

### RIPng Configuration Summary

| Router | Command | Purpose |
| --- | --- | --- |
| All Routers | ipv6 unicast-routing | Enable IPv6 routing globally |
| All Routers | ipv6 router rip RIPNG | Create RIPng process |
| Per Interface | ipv6 rip RIPNG enable | Enable RIPng on interface |

## 10. PC Configuration

### PC0 Configuration

| Setting | Value |
| --- | --- |
| IPv6 Address | 2001:DB8:1:0:2E0:B0FF:FE86:5EA7 |
| Link Local Address | FE80::2E0:B0FF:FE86:5EA7 |
| Default Gateway | FE80::204:9AFF:FEA2:EE01 |
| Configuration Method | Autoconfigure |

### PC1 Configuration

| Setting | Value |
| --- | --- |
| IPv6 Address | 2001:DB8:2:0:20A:41FF:FEE2:D773 |
| Link Local Address | FE80::20A:41FF:FEE2:D773 |
| Default Gateway | FE80::204:9AFF:FEA2:EE01 |
| Configuration Method | Autoconfigure |

### PC2 Configuration

| Setting | Value |
| --- | --- |
| IPv6 Address | 2001:DB8:3:0:2E0:A3FF:FE06:6410 |
| Link Local Address | FE80::2E0:A3FF:FE06:6410 |
| Default Gateway | FE80::204:9AFF:FEA2:EE01 |
| Configuration Method | Autoconfigure |

## 11. Testing and Verification

The following tests were performed successfully:

### Test 1: Verify Router Interfaces

Command: show ip interface brief

| Router | Interface | Status | Protocol |
| --- | --- | --- | --- |
| R0 | Gig0/0/0 | up | up |
| R0 | Serial0/1/0 | up | up |
| R1 | Serial0/1/0 | up | up |
| R1 | Serial0/1/1 | up | up |
| R2 | Gig0/0/0 | up | up |
| R2 | Serial0/1/0 | up | up |

### Test 2: Verify IPv6 Routing Table (R0)

Command: show ipv6 route

### Test 3: Ping Test from PC0 to PC1

Command: ping 2001:DB8:2:0:20A:41FF:FEE2:D773

### Test 4: Ping Test from PC2 to PC1

Command: ping 2001:DB8:3:0:2E0:A3FF:FE06:6410

## 12. Troubleshooting

Some issues encountered during implementation and their solutions:

| # | Issue | Solution |
| --- | --- | --- |
| 1 | Serial ports not visible on ISR 4321 | Added NIM-2T module in Physical tab |
| 2 | Serial interface showing "down/down" | Used no shutdown command on both ends |
| 3 | Clock rate not set | Configured clock rate 64000 on DCE end |
| 4 | No IPv6 routes in routing table | Verified ipv6 unicast-routing was enabled |
| 5 | RIPng not sharing routes | Ensured ipv6 rip RIPNG enable on all interfaces |
| 6 | PC IPv6 address not saving | Used Autoconfigure method in IP Configuration |
| 7 | Ping showing "Destination host unreachable" | Verified default gateway (FE80::204:9AFF:FEA2:EE01) on PCs |
| 8 | Serial ports on different numbers (0/1/0 vs 0/0/0) | Checked physical module slot placement |

Key Learning: Always verify interface status using show ip interface brief before testing connectivity. IPv6 link-local addresses are used as default gateways in IPv6 networks.

## 13. Conclusion

This project successfully demonstrated the implementation of an IPv6-based routed network using RIPng as the dynamic routing protocol in Cisco Packet Tracer.

The network allowed communication between devices in different IPv6 subnets:

- PC0 (2001:DB8:1:0:2E0:B0FF:FE86:5EA7) successfully communicated with PC1 (2001:DB8:2:0:20A:41FF:FEE2:D773)

- PC0 successfully communicated with PC2 (2001:DB8:3:0:2E0:A3FF:FE06:6410)

RIPng was successfully configured to automatically share route information between all three routers, eliminating the need for manual static route configuration. Serial DCE links were properly configured with clock rates to simulate real-world WAN connections.

The project provided practical understanding of:

- IPv6 addressing and subnetting

- RIPng dynamic routing protocol configuration

- Serial WAN link implementation

- Network troubleshooting using show and ping commands

- Cisco IOS command-line interface

- IPv6 link-local addresses as default gateways

This project serves as a foundation for understanding modern IPv6-based enterprise networks and the transition from IPv4 to IPv6.

## 14. References

| # | Reference |
| --- | --- |
| 1 | Cisco Networking Academy. (2024). IPv6 Fundamentals and RIPng Configuration |
| 2 | Cisco Packet Tracer Documentation |
| 3 | RFC 2080 - RIPng for IPv6 |
| 4 | RFC 8200 - Internet Protocol, Version 6 (IPv6) Specification |
| 5 | Cisco Systems. (2023). Configuring RIPng on Cisco Routers |
| 6 | Computer Networking Laboratory Manual - IPv6 Routing |